DLCD -ASSIGNMENT

PREETHAM s

PES2UG25CS405

G SECTION

1.Dffresetload.v

module dffresetload (

input wire clk,

input wire reset,

input wire load,

input wire in,

output reg out

);

always @(posedge clk) begin

if (reset) begin

out <= 1'b0;

end else if (load) begin

out <= in;

end else begin

out <= out;

end

end

endmodule

DFFRESetload_tb.v

`timescale 1ns / 1ps

module dffresetload_tb;

reg clk;

reg reset;

reg load;

reg in;

wire out;

// Instantiate the D Flip-Flop module

dffresetload uut (

.clk(clk),

.reset(reset),

.load(load),

.in(in),

.out(out)

);

// Display formatted simulation output

initial begin

$monitor("clk=%b, reset=%b, load=%b, in=%b, out=%b", clk, reset, load, in, out);

end

// VCD file dump for GTKWave

initial begin

$dumpfile("dff_test.vcd");

$dumpvars(0, dffresetload_tb);

end

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

reset = 1; load = 0; in = 1;

#10;

reset = 0; load = 0; in = 1;

#5;

load = 1; in = 1;

#5;

in = 0;

#10;

load = 0; in = 1;

#5;

in = 0;

#5;

$finish;

end

endmodule

2.Dff.v

module dff (

input wire clk,

input wire in,

output reg out,

output wire out_bar

);

assign out_bar = ~out;

always @(posedge clk) begin

out <= in;

end

endmodule

Dff_tb.v

`timescale 1ns / 1ps

module dff_tb;

reg clk;

reg in;

wire out;

wire out_bar;

dff uut (

.clk(clk),

.in(in),

.out(out),

.out_bar(out_bar)

);

initial begin

$monitor("%0dclk=%b, in=%b, out=%b", $time, clk, in, out);

end

// VCD dump for GTKWave visualization

initial begin

$dumpfile("dff_test.vcd");

$dumpvars(0, dff_tb);

end

// Clock generation (Period = 10ns)

initial begin

clk = 0;

forever #5 clk = ~clk;

end

// Stimulus matching the exact timing sequence (0ns to 40ns)

initial begin

// Time 0ns: initial setup

in = 1;

#5;  // Time 5ns (posedge clk) -> in=0

in = 0;

#5;  // Time 10ns (negedge clk) -> in=1

in = 1;

#10; // Time 20ns (negedge clk) -> in=0

in = 0;

#10; // Time 30ns (negedge clk) -> in=1

in = 1;

#10;

in = 0;

#0;

$finish;

end

endmodule

3.Dffreset.v

module dffreset (

input wire clk,

input wire reset,

input wire in,

output reg out

);

always @(posedge clk) begin

if (reset) begin

out <= 1'b0;

end else begin

out <= in;

end

end

endmodule

Dffreset_tb.v

`timescale 1ns / 1ps

module dffreset_tb;

reg clk;

reg reset;

reg in;

wire out;

dffreset uut (

.clk(clk),

.reset(reset),

.in(in),

.out(out)

);

initial begin

$monitor("%0dclk=%b,reset=%b, in=%b, out=%b", $time, clk, reset, in, out);

End

initial begin

$dumpfile("dff_test.vcd");

$dumpvars(0, dffreset_tb);

end

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

reset = 1; in = 1;

#5;  // Time 5ns (posedge clk) -> in=0

in = 0;

#5;  // Time 10ns (negedge clk) -> in=1

in = 1;

#3;  // Time 13ns -> reset drops to 0

reset = 0;

#2;  // Time 15ns (posedge clk) -> in=1

#5;  // Time 20ns (negedge clk) -> in=0

in = 0;

#10; // Time 30ns (negedge clk) -> in=1

in = 1;

#10; // Time 40ns (negedge clk) -> in=0

in = 0;

#0;  // End simulation at 40ns

$finish;

end

endmodule